Leitrim County Council

Winter Service Maintenance Plan

2016 – 2017

###### Document Control

| Revision | Status | Revision Details | Date |
| --- | --- | --- | --- |
|  |  |  | October 2010 |
| Rev A |  | Updated | Sept 2011 |
| Rev B |  | Updated | Sept 2012 |
| Rev C |  | Updated | Sept 2013 |
| Rev D |  | Updated | Sept 2014 |
| Rev E |  | Updated | Sept 2015 |
| Rev F |  | Updated | Oct 2016 |

###### Approval List

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Author | Terry McGovern S.E.E. |  |  |
| Checker | Shay O’ Connor S.E |  |  |
| Winter Service Manager | Shay O’ Connor S.E. |  |  |
| Director of Services for Transportation | Joseph Gilhooly |  |  |

Distribution List

| Issued To | Organisation / Company | Location |
| --- | --- | --- |
| Duty Engineers | Leitrim Co. Council |  |
| Area Engineers | Leitrim County Council |  |
| Winter Service Supervisor(s) | Leitrim County Council |  |
| Health and Safety Officer | Leitrim County Council |  |
| Department Of Transport | Department of Transport |  |
| Winter Maintenance Manager | NRA |  |
| Director of Service for Transportation | Sligo, Roscommon, Longford, Cavan and Donegal Co. Cos. |  |
| Winter Maintenance Manager, Western Division | Roads Service NI | County Hall, Drumragh Avenue Omagh, Co. Tyrone BT 79 7AF |
| Chief Superintendent | Sligo – Leitrim Garda District | Sligo Garda Station. |
| Fire Service | Chief Fire Officer, Leitrim |  |
| Ambulance Service | HSE |  |
| Head of Forecasting | Met Éireann | Glasnevin Hill, Dublin 9 |

###### Purpose and Scope

The purpose of this document is to identify the processes, procedures and key personnel employed by Leitrim County Council, to deliver the winter service for County Leitrim. This plan may be amended as necessary during the winter season. The winter season runs from 17th October 2016 to the 30th April 2017. This plan has been prepared in consultation with the Draft Winter Service Manual published in 2010 by the National Roads Authority (NRA) and the City & County Managers Association (CCMA).

###### Policy

There are approximately 2,500 km of public roads in county Leitrim. For practical and financial reasons only a certain portion of the road networks can be included in the Winter Service Plan. It is therefore necessary to prioritise important and strategic routes in order to use available resources to the greatest benefit.

The following is Leitrim County Councils policy with regard to the winter service, including the description of and level of service on route Priorities 1, 2 & 3, and the treatment and mobilisation times.

| Route Designation | Description |  |  | Level of Service |  | Routes in Co. Leitrim |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Priority 1 | Those routes which are essential to be kept serviceable in all weather conditions, as far as reasonably practicable |  |  | To be treated during all weather events |  | N4, N15, N16 (from Sligo Co. Bdry to Fermanagh Co. Bdry) |  |
| Priority 2 | Those routes which are desirable to be kept serviceable in normal weather conditions, as far as reasonably practicable |  |  | To be treated as part of the normal winter service but may have interruptions to treatment in certain severe weather events |  | R280,(incl. section in Co. Roscommon) R371, LP 1601-1, R299, LP 3408, R202, R199, R205, R201, R203, R207, R208, R200, R209 Part (Letterfine to R280) R210 Part (Letterfine to Drumcong) R289 (R280 to Dromahaire) R287 (Dromahaire to R280) R286 R287 (To be carried out by Sligo Co Co) R281 (Glenfarne to Rossinver) R282 (Rossinver to Manorhamilton) |  |
| Priority 3 | Those routes that could be kept serviceable once Priority 1 and 2 routes have been treated, if resources allow. |  |  | Not treated as part of the normal winter service but may receive intermittent treatment during certain severe weather events |  | All other Regional Roads and Priority County Roads. |  |
| Treatment Route |  | Priority 1 | Priority 2 |  | Priority 3 |  |  |
| Mobilisation Time |  | 1 hr | 1 hr |  | When possible |  |  |
| Treatment Time |  | 3 hr | 4 hrs |  | When possible |  |  |

Footpaths

Leitrim County Council will, during severe weather, clear snow/ice from footpaths at strategic locations in the towns and villages. Property owners are encouraged to clear snow from the footpaths outside their property. We will advise the public of the best equipment to use and the safest way to use this equipment in clearing snow from footpaths.

###### Overall Map of the Service Plan Area

See Maps at Appendix A

###### Weather Monitoring and the Treatment Decision process

Leitrim County Council appoints Duty Engineers to review the weather conditions and make a decision on the treatment to be carried out, each day during the winter period. The weather predictions are made available to Leitrim County Council through the NRA web based weather application provided by Met Eireann and Vaisala. The Duty Engineer will monitor the weather conditions as necessary and alter any decisions made as required.

Winter Service Duty Engineers and Winter Service Supervisors must attend to their normal Council duties during normal Council working hours and be competent to do so. Accordingly, Leitrim County Council acknowledges that there is a limit to the extent of overnight monitoring that can be expected of a Duty Engineer. In particular, the Council would expect that treatment decisions would normally be communicated to the Winter Service Supervisor before 22:00 hours. If the treatment decision is deferred to a later time, the Duty Engineer should advise the Winter Service Supervisor of the deferred treatment decision deadline at or prior to 22:00 hours.

Details of the rosters for the duty engineers in Appendix B.

###### Contact Details

| Duty Forecaster Met Eireann | (01) 8065550 |
| --- | --- |
| Technical Support for Bureau Weather Service | (01) 8064219 or (01) 8064255 |
| Garda Point of Contact | (071)9142031 or (071)9157000 |
| Winter Service Manager Donegal County Council |  |
| Winter Service Manager Sligo County Council |  |
| Winter Service Manager Roscommon County Council |  |
| Winter Service Manager Longford County Council |  |
| Winter Service Manager Cavan County Council |  |
| Winter Service Manager Western Division, Road Service NI | 00 44 28 82254111 or 00 44 28 82241999 |
| Commisky Plant Hire | 087 9683006 |
| Ambulance Service | 999 or 112 |
| Fire Service | 999 or 112 |

###### Supplementary Resources

Details of resources available to supplement the winter service are included in Appendix C

Decision Matrix

The following table is our Decision Matrix for treatment of roads in severe weather conditions.

| Road Surface Temperature | Precipitation | Predicted Road Conditions |  |  |
| --- | --- | --- | --- | --- |
|  |  | Wet | Wet Patches | Dry |
| May fall below 1°C | No rain No hoar frost No fog | Salt before frost | Salt before frost (see note a) | No action likely, monitor weather (see note a) |
| Expected to fall below 1°C | No rain No hoar frost No fog |  |  |  |
|  | Expected hoar frost Expected fog |  | Salt before frost (see note b) |  |
|  | Expected rain before freezing | Salt after rain stops (see note c) |  |  |
|  | Expected rain during freezing | Salt before frost, as required during rain and after rain stops (see note d) |  |  |
|  | Possible rain Possible hoar frost Possible fog | Salt before frost |  | Monitor weather conditions |
| Expected Snow |  | Salt before snow fall |  |  |
| The decision to undertake precautionary treatments should be, if appropriate, adjusted to take account of residual salt or surface moisture. All decisions should be evidence based, recorded and require careful monitoring and review. |  |  |  |  |

Notes:

- Particular attention should be given to the possibility of water running across carriageways and other running surfaces e.g. surface water off adjacent fields after heavy rains, washing off salt previously deposited.  Such locations should be closely monitored and may require treating in the evening and morning and possible other occasions.  Ideally the source of the run-off should be diverted from the roadway.

- When a weather warning contains reference to expected hoar frost, considerable deposits of frost can occur.  Hoar frost usually occurs in the early morning and is difficult to cater for because of the probability that any salt deposited on a dry road too soon before its onset, may be dispersed before it can become effective.  Careful monitoring is required under this forecast condition which should ideally be treated just as the hoar frost is forming.  Such action is usually not practicable and salt may have to be deposited on a dry road prior to but as close as possible to the expected time of the condition.  Hoar frost may also be forecast at other times of the day, in which case the timing of salting operations should be adjusted accordingly.

- If under these conditions, rain has not ceased by early morning, crews should be called out and action initiated as rain ceases.

- Under these circumstances rain will freeze on contact with running surfaces and full precautionary treatment should be provided even on dry roads.  This is a most serious condition and should be monitored closely and carefully throughout the danger period.

###### Treatment Matrix

The following table is our Treatment Matrix for the various weather conditions.

Treatment Matrix Guide

| Weather Conditions Road Surface Conditions Road Surface Temperature (RST) | Treatment |  |  |
| --- | --- | --- | --- |
|  | Air Temperature | Salt Spread Rate (gm/m2) | Ploughing |
| Frost or forecast frost RST at or above -2°C |  | 10/15 | No |
| Frost or forecast frost RST below -2°C and above -5°C |  | 20 | No |
| Frost or forecast frost RST below -5°C and above -10°C and dry or damp road conditions |  | 20 | No |
| Frost or forecast frost RST below -5°C and above -10°C and wet road conditions (existing or anticipated) |  | 2 runs X 20 | No |
| Light snow forecast (<10mm) |  | 20 | No |
| Forecast for Medium/heavy snow or a freezing rain forecast |  | 2 runs X 20 | No |
| Ice formed | Above -5°C | 20 | Not possible |
| Ice formed | At or below -5°C | 2 runs X 20 | Not possible |
| Snow covering exceeding 30mm |  | 20 to supplement ploughing, up to 40 if temperatures are falling | Required |
| Snow accumulations due to prolonged falls |  | 20 to 40 to supplement ploughing | Required |
| Hard packed snow/ice | Above -8°C | Successive treatments at 20 to 40 (repeat as needed) | Not possible |
| Hard packed snow/ice | At or below -8°C | Successive treatments at 20 to 40, supplemented by abrasives (repeat as needed) | Not possible |
| Sustained low temperatures | Below -10°C | Refer to Roads Section |  |

Notes:

- Rate of spread for precautionary treatments may be adjusted to take account of variations occurring along the route such as residual salt, surface moisture (in the air or on the road surface) and traffic density.

- All decisions should be forecast based, recorded and require careful monitoring and review.

- Ice refers to all ice on the road surface, including black ice.

- For salt stored outside it may be necessary to increase spread rate for precautionary treatment salting from 10gm/m2 to 15gm/m2. Duty engineer to decide which rate to use.

Target Spread Rates by Location

| Location | Salt Spread Rate |
| --- | --- |
| Carriageways | As described in the Treatment Matrix Guide |
| Hard shoulder or carriageway marginal strips | 50% of selected treatment |
| Porous Surfacing | Plus 25% of selected treatment |
| Footways, cycle tracks and pedestrian areas | 25 gm/m2 |

###### Plant, Vehicles and Equipment

Leitrim County Council operates 5 number demountable mounted salt spreaders, and 6 number snowploughs, to deliver the winter service. We also have access to other hire equipment if the situation requires.

| Description | Depot Location | Capacity | Date Calibrated | Date Serviced | Date of next Calibration | Date of next Service |
| --- | --- | --- | --- | --- | --- | --- |
| Plant No 39 Demountable Salt Spreader | Faulties | 9 m3 | Sept 2016 | Sept 2016 | Sept 2017 | Sept 2017 |
| Plant No. 24 - Demountable salt spreader | Faulties | 5 m3 | Sept 2016 | Sept 2016 | Sept 2017 | Sept 2017 |
| Plant No. 25 - Demountable salt spreader | Faulties | 5 m3 | Sept 2016 | Sept 2016 | Sept 2017 | Sept 2017 |
| Plant No. 29 - Demountable salt spreader | Manorhamilton | 6 m3 | Sept 2016 | Sept 2016 | Sept 2017 | Sept 2017 |
| Plant No. 30 - Demountable salt spreader | Manorhamilton | 6 m3 | Sept 2016 | Sept 2016 | Sept 2017 | Sept 2017 |
| Demountable Mini Spreader | Mohill | 2 m3 | Sept 2012 | Sept 2012 | Sept 2017 | Sept 2017 |

| Snowploughs & Loading Shovels | Depot Location | Fitted to | Date Serviced | Date of next Service |
| --- | --- | --- | --- | --- |
| Snowplough | Faulties |  | Oct 2016 |  |
| Plant No 97A – Snowplough | Faulties | 01LM1136 Plant 28 | Oct 2016 | Sept 2017 |
| Plant No 98 – Snowplough | Manorhamilton | 11LM435 Plant 36 | Oct 2016 | Sept 2017 |
| Plant No 99 – Snowplough | Faulties | 03LM822 Plant 31 | Oct 2016 | Sept 2017 |
| Plant No 97D – Snowplough | Manorhamilton | 06LM1334 Plant 33 | Oct 2016 | Sept 2017 |
| Plant No 18A – Snowplough | Faulties | 97LM874 Plant 18 | Oct 2016 | Sept 2017 |
| Plant No 32A - Snowplough | Dromahaire | 05LM1053 Plant 32 | Oct 2016 | Sept 2017 |
| Plant No 12 Landini Tractor Loading Shovel | Faulties | 06RN3615 |  |  |
| Plant No 13 Landini Tractor Loading Shovel | Dromahaire | 04RN1876 |  |  |

Note: Duty Engineers will advise when demountable gritters are to be mounted before weekends and holiday periods.

###### Details of Depots

| Map Ref. | Salt Depots | Routes Serviced | Indoor or Outdoor | Max Capacity | Minimum stock levels at: |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 1st Oct | 15th Dec | 1st Mar |
|  | Faulties | N4, R202, R280, R207, R208. | Indoor | 2000 Tonnes | 100 Tonnes |  |  |
|  | Ballinamore | R199, R201, R202, R205, R208, R209, R210. | Indoor | 300 Tonnes | 90 Tonnes |  |  |
|  | Manorhamilton | N15 N16 R280 | Indoor | 300 Tonnes | 150 Tonnes |  |  |
|  | Drumkeeran | R280, R289, R287, R207, R200 | Indoor | 300 Tonnes | 120 Tonnes |  |  |
|  | Mohill | Local Roads | Outdoor | 50 Tonnes | 0 Tonnes |  |  |
|  |  | TOTALS |  | 2950 Tonnes | 460 Tonnes |  |  |

###### Materials and Spares

The Area Engineers will be responsible for re-ordering de-icing materials. The new procurement in operation by the NRA will be used for procuring salt this winter.

There are small quantities of fuel stored at each Depot for emergencies, but the majority of the fuel will be acquired at Retail outlets.

Spare parts for winter service machinery are stored in the machinery yard. The Machinery Yard supervisor will be responsible for reordering stock.

Community Involvement

During severe weather conditions Leitrim County Council will make available grit at 36 locations throughout the county to enable people in the community to use it to treat local roads, hills etc that we are never going to be able to get to due to the demand on our resources. See Appendix D for map and a full list of the locations of the drop points. This grit is available for any person to use. They will not be working for Leitrim County Council when spreading this grit.

Leitrim County Council will introduce a scheme where residents associations and business can purchase from the council a salt bin and salt for their housing estates or business premises. They will be charged a nominal fee. Depending on our resources at the time the bin will be refilled by us or salt made available to the residents at our depot, for a nominal fee.

APPENDIX   A

APPENDIX   B

APPENDIX   C

APPENDIX   D

Grit Locations – County Leitrim

Ball Room of Romance Glenfarne N16

Rossinver Football Pitch

Scollans Depot, Largydonnell

NCF Laughta Road, Kinlough

Askill School

Lough Doon, Fivemilebourne

St Michael Church, Barrs West

Kiltyclogher - Old Landfill Site

Coraleehan Church

Cromlin Cross Roads

Greaghglass, Pol an Easa Amenity

Leitrim County Council  Depot, Ballinamore Depot

Drumeela church.

Corawallen P.O.

Corriga Reservoir

Glan Cross Roads, Beagh More

Rassuan Church

Nailers Cross Roads, Feraglass North

Cornageeha Church

Greagh Cross Roads, Lissagarvan

Gubbs Crossroads, Corduff

District Hospital, Summerhill, Carrick on Shannon

Kilboderry, Carrick on Shannon

Kiltoghert Community Centre

Leitrim County Council Depot, Faulties

Mohill Church

Bornacoola Church

Eslin Church

Gortlettragh Church

Fearglass Church

Annaduff Church

Crossroads Toomans

Aughacashel

Leitrim County Council Depot Kildoon

Drumkeeran Old Landfill Site

Ballinaglera Village

Aughavas

Carrigallen Mart